20EE001

Hall Ticket Number:

| I/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| April, 2022 | Common to CB,DS, IT and ME |  |  |
| First Semester | Basic Electrical & Electronics Engineering |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer Question No.1 compulsorily. |  |  | (1X14 = 14 Marks) |
| Answer ONE question from each unit. |  |  | (4X14=56 Marks) |

| 1 | Answer all questions |  |  | (1X14=14 Marks) |  |
| --- | --- | --- | --- | --- | --- |
|  | a) |  | State kirchhoff’s Voltage law. |  |  |
|  | b) |  | What are the voltage – current relationships in electrical circuit elements? |  |  |
|  | c) |  | How the rotating magnetic field is produced in three phase induction motors. |  |  |
|  | d) |  | Draw torque – slip characteristics of three phase induction motor. |  |  |
|  | e) |  | Mention various loses in DC machine. |  |  |
|  | f) |  | Write the impedance of an RL series circuit and write its phasor diagram. |  |  |
|  | g) |  | Draw the symbol of P-N-P transistor |  |  |
|  | h) |  | Write two applications of Transistor. |  |  |
|  | i) |  | What are different types of semiconductors? |  |  |
|  | j) |  | What is clamper? |  |  |
|  | k) |  | Differentiate between BJT and JFET. |  |  |
|  | l) |  | Define CMRR |  |  |
|  | m) |  | Compare a practical op-amp to ideal op-amp? |  |  |
|  | n) |  | Draw the unity follower circuit using Op-Amp |  |  |
| UNIT I |  |  |  |  |  |
| 2. |  | a) | Apply Thevenin’s theorem to find Vo in the circuit of Fig. |  | 7M |
|  |  | b) | State and explain KVL and KCL with examples. |  | 7M |
| (OR) |  |  |  |  |  |
| 3. |  | a) | Derive the relation between line voltage and phase voltage, line current and phase current in three phase star connected system with phasor diagram. |  | 7M |
|  |  | b) | Using nodal analysis, find vo in the circuit of Fig. |  | 7M |
| UNIT II |  |  |  |  |  |
| 4. |  | a) | Explain the working principle of a DC machine |  | 7M |
|  |  | b) | Explain the construction of single phase transformer and write the losses in transformer. |  | 7M |
| (OR) |  |  |  |  |  |
| 5. |  | a) | Draw and explain the torque-slip characteristics of three phase induction machine. |  | 7M |
|  |  | b) | Discuss about the single phase capacitor motor operation. |  | 7M |
| UNIT III |  |  |  |  |  |
| 6. |  | a) | Explain the operation of PN junction diode in forward and reverse bias condition with V-I characteristics. |  | 7M |
|  |  | b) | Explain the operation of NPN transistor in CE configuration. |  | 7M |
| (OR) |  |  |  |  |  |
| 7. |  | a) | Draw the circuit diagram of a full wave rectifier and explain its operation. |  | 7M |
|  |  | b) | Explain the operation of Zener diode as voltage regulator. |  | 7M |
| UNIT IV |  |  |  |  |  |
| 8. |  | a) | Explain the operation of clippers and clampers. |  | 7M |
|  |  | b) | Explain the characteristics of JFET |  | 7M |
| (OR) |  |  |  |  |  |
| 9. |  | a) | Explain the operation of Op-Amp as non-inverting amplifier. |  | 7M |
|  |  | b) | Explain the operation of Op-Amp as an integrator with a circuit diagram |  | 7M |