Hall Ticket Number:                                                                                        20EE205

| I/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| August, 2023 | Electrical and Electronics Engineering |  |  |
| Second Semester | CIRCUIT THEORY |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | State Ohm’s law. | CO1 | L2 | 1 |
|  | b) | Define passive circuit, give an example. | CO1 | L1 | 1 |
|  | c) | Define form factor | CO1 | L1 | 1 |
|  | d) | What are advantages of sinusoidal waveform in electrical systems? | CO2 | L1 | 1 |
|  | e) | Define mesh | CO2 | L1 | 1 |
|  | f) | Define Supernode | CO2 | L1 | 1 |
|  | g) | What is meant by phase difference? | CO3 | L1 | 1 |
|  | h) | Draw the phasor diagram for series RL circuit | CO3 | L2 | 1 |
|  | i) | State compensation theorem. | CO4 | L2 | 1 |
|  | j) | Write the condition for maximum power in maximum power transfer theorem. | CO4 | L2 | 1 |
|  | k) | State Tellegen’s theorem. | CO1 | L2 | 1 |
|  | l) | What is the value of impedance at resonance? | CO1 | L1 | 1 |
|  | m) | What is Band width in series resonance? | CO1 | L1 | 1 |
|  | n) | Define Q-factor of an electrical network. | CO2 | L1 | 1 |
| Unit-I |  |  |  |  |  |
| 2 | a) | State and Explain KVL & KCL with an example. | CO1 | L2 | 7M |
|  | b) | Find the source current in the resistance network shown below by using star-delta transformation. | CO1 | L1 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | Find average & RMS values of the waveform shown below. | CO1 | L1 | 7M |
|  | b) | An impedance Z1= (6+j8) Ω is connected in series with a parallel combination of impedances Z2= (10+j6) Ω, Z3= (8-j10) Ω and is connected to a 300V, 50Hz supply. Find the total active power, reactive power & power factor of the circuit. | CO1 | L2 | 7M |
| Unit-II |  |  |  |  |  |
| 4 | a) | What is the magnitude of current drained from the 10V source in the given circuit? | CO2 | L1 | 7M |
|  | b) | Determine V in the circuit shown in figure. P.T.O | CO2 | L3 | 7M |
| 20EE205 (OR) |  |  |  |  |  |
| 5 | a) | - Explain RL series circuit with pulse excitation. | CO2 | L2 | 7M |
|  | b) | Find the power absorbed by each element for the given circuit. | CO2 | L1 | 7M |
| Unit-III |  |  |  |  |  |
| 6 | a) | State Thevenin’s theorem. Write any two advantages of this theorem. | CO3 | L1 | 4M |
|  | b) | State and prove Maximum power transfer theorem for AC and DC excitations | CO3 | L2 | 10M |
| (OR) |  |  |  |  |  |
| 7 | a) | Find the current through 2Ω resistor for given circuit using Norton’s theorem. | CO3 | L1 | 7M |
|  | b) | Find voltage across 10Ω resistance in the network shown below by using Superposition theorem. | CO3 | L2 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Derive an expression for Bandwidth of Series RLC circuit. | CO4 | L3 | 7M |
|  | b) | An RLC Series circuit consists of R=1kΩ, L=100mH, C=10µF. If a voltage of 100V is applied across the combination, determine resonant frequency, quality factor and bandwidth. | CO4 | L2 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | Find the resonant frequency for the given circuit. | CO4 | L1 | 7M |
|  | b) | Deduce the current locus diagram of RL series circuit with L as variable parameter. | CO4 | L3 | 7M |