| 20ME303 Hall Ticket Number: II/IV B.Tech( Regular/Supplementary) DEGREE EXAMINATION. February,2023; Mechanical Engineering. Third Semester; Strength of Materials. Time: Three Hours; Maximum:70 Marks. |  |
| --- | --- |
| Answer question 1 compulsory.; (14X1 = 14 Marks). Answer one question from each unit.; (4X14=56 Marks). |  |

| 1. | a) |  | Classify the different types stresses. | CO1 | L1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | b) |  | Sketch the bar of varying section subjected to axial load. | CO1 | L2 |  |
|  | c) |  | Differentiate between Twisting moment and Bending Moment. | CO1 | L3 |  |
|  | d) |  | What is the yield stress of Fe415 grade steel. | CO2 | L1 |  |
|  | e) |  | Differentiate between long and short column. | CO2 | L3 |  |
|  | f) |  | What are the limitations of Euler’s theory? | CO2 | L1 |  |
|  | g) |  | What do you mean by beam-column? | CO3 | L1 |  |
|  | h) |  | Define core of section. | CO3 | L1 |  |
|  | i) |  | What do you mean by principal stress? | CO3 | L1 |  |
|  | j) |  | Show the location of neutral axis for a symmetric I-section with help of neat sketch. | CO4 | L2 |  |
|  | k) |  | What are the limitations of Mohrs circle. | CO4 | L1 |  |
|  | l) |  | What is the difference between Zero bending moment and point of contraflexure | CO4 | L1 |  |
|  | k) |  | List the assumption in bending theory. | CO1 | L1 |  |
|  | l) |  | Give the limitation of Moment area method. | CO2 | L1 |  |
|  |  | Unit –I |  |  |  |  |
| 2. | a) |  | Sketch stress-strain diagram for HYSD steel and indicate the salient points on it. | CO1 | L2 | 4M |
|  | b) |  | A rod has diameter of 120mm at one end and tapers uniformly to a diameter of 60mm over a length of 400mm. Find the elongation of the rod, when it is subjected to a load of 200kN. E = 200GPa. | CO1 | L4 | 10M |
|  |  | (OR) |  |  |  |  |
| 3. | a) |  | Discuss the axial deformation of a bar of varying section. | CO1 | L2 | 4M |
|  | b) |  | The steel bolt shown in Figure has a thread pitch of 1.6 mm. If the nut is initially tightened up by hand so as to cause no stress in the copper spacing tube, calculate the stresses induced in the tube and in the bolt if a spanner is then used to turn the nut until the compressive load acting 20KN.Take Ec and Es as 100 GPa and 209 GPa respectively. Fig.1 | CO1 | L3 | 10M |
|  |  | Unit –II |  |  |  |  |
| 4. | a) |  | Derive the Torsion equation. | CO2 | L1 | 7M |
|  | b) |  | A solid circular shaft transmits power 75kW power at 200rpm. Estimate the Values of shaft diameter, if the twist in the shaft is not to exceed one degree in 2m length of shaft and shear strength is not to exceed 50N/mm2. Take G=100kN/mm2. | CO2 | L1 | 7M |
|  |  | (OR) P.T.O 20ME303 |  |  |  |  |
| 5. | a) |  | Distinguish between the point of contra flexure and point of zero bending moment at a beam section | CO2 | L3 | 6M |
|  | b) |  | A Overhanging beam shown in fig.2. Draw shear force and bending moment diagrams. Show the significant values. Fig.2 | CO2 | L4 | 8M |
|  |  | Unit –III |  |  |  |  |
| 6. | a) |  | Describe what do you understand by “the material is homogeneous , Isotropic and obeys Hooks law” which is one of the assumption in theory of simple bending. | CO3 | L2 | 4M |
|  | b) |  | A beam of rectangular section of 200 mm deep and 300 mm wide is simply supported over a span of 8 m. What uniformly distributed load per meter the beam may carry, if the bending stress not to exceed 120 Mpa. | CO3 | L4 | 10M |
|  |  | (OR) |  |  |  |  |
| 7. | a) |  | Discuss the advantages of Macaulay’s method over Double integration method. | CO3 | L2 | 4M |
|  | b) |  | A simply supported beam of span ‘L’ is subjected to loads as shown in the fig.3. Determine the maximum deflection and slope by using moment area method. Fig.3 | CO3 | L3 | 10M |
|  |  | Unit –IV |  |  |  |  |
| 8. | a) |  | Explain the significance of Mohrs Circle. | CO4 | L2 | 6M |
|  | b) |  | At a point in a strained material, there are normal stresses of 80Mpa (Compressive) and 50Mpa( Tensile) at right angles to each other with a shear stress 30MPa( negative). Determine the principal stresses, maximum shear stress and plane on which they act. Show them on the sketches of properly oriented elements. Use Mohr’s circle method. | CO4 | L4 | 8M |
|  |  | (OR) |  |  |  |  |
| 9. | a) |  | Derive the Eulers buckling load for a column with both ends are fixed | CO4 | L3 | 6M |
|  | b) |  | A piston rod of steam engine 80 cm long is subjected to a maximum load of 60 kN. Determine the diameter of the rod using Rankine’s formula with permissible compressive stress of 100 N/mm2. Take constant in Rankines formula as 1/7500 for hinged ends. The rod may be assumed partially fixed with length coefficient of 0.6. | CO4 | L4 | 8M |