20ME404

Hall Ticket Number:

| II/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July/August, 2023 | Mechanical Engineering |  |  |
| Fourth Semester | Applied Thermodynamics |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |
|  |  |  | CO BL M |

| 1 | a) | What are saturation states? | CO1 | L1 | 1M |
| --- | --- | --- | --- | --- | --- |
|  | b) | What do you understand by the degree of superheat? | CO1 | L2 | 1M |
|  | c) | Draw the Rankine cycle on T-S diagram. | CO1 | L1 | 1M |
|  | d) | What is the effect of steam reheat on the cycle efficiency of a steam power plant? | CO1 | L2 | 1M |
|  | e) | Define a boiler. | CO2 | L1 | 1M |
|  | f) | What is the function of a safety valve? | CO2 | L1 | 1M |
|  | g) | List the different types of boiler accessories. | CO2 | L1 | 1M |
|  | h) | What is the basic function of nozzle in steam power plants? | CO2 | L1 | 1M |
|  | i) | List the functions of the steam condenser in a thermal power plant | CO3 | L1 | 1M |
|  | j) | Define condenser efficiency. | CO3 | L1 | 1M |
|  | k) | What is the compounding of a steam turbine? | CO3 | L1 | 1M |
|  | l) | Define COP of a refrigerator | CO4 | L1 | 1M |
|  | m) | Define the Relative humidity and degree of saturation. | CO4 | L2 | 1M |
|  | n) | How to represent dehumidification with heating on Psychometric Chart. | CO4 | L2 | 1M |
| Unit-I |  |  |  |  |  |
| 2 | a) | Draw the p v diagram of a pure substance and explain how it formed. | CO1 | L1 | 7M |
|  | b) | A rigid vessel of volume 0.86 m3 contains 1 kg of steam at a constant pressure of 2 bar. Evaluate the specific volume, temperature, dryness fraction, enthalpy, entropy and internal energy of steam. | CO1 | L2 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | Explain the working principle of the Reheat cycle with a suitable neat sketch. | CO1 | L1 | 7M |
|  | b) | A simple Rankine cycle works between pressures 30 bar and 0.08 bar. The initial condition of steam is dry saturated. Calculate the cycle efficiency, work ratio and Specific steam consumption. | CO1 | L2 | 7M |
| Unit-II |  |  |  |  |  |
| 4 | a) | How are boilers classified? | CO2 | L1 | 7M |
|  | b) | What are the functions of the following in a boiler (i) superheater (ii) Feed check valve (iii) blow-off cock valve. | CO2 | L1 | 7M |
| (OR) |  |  |  |  |  |
| 5 | a) | Derive the expression for Critical pressure ratio in nozzles. | CO2 | L2 | 7M |
|  | b) | Dry saturated steam expands through a nozzle from a pressure of 13.5 bar to 9.6 bar. Assuming the flow to be frictionless and adiabatic, estimate the velocity of the steam jet. | CO2 | L2 | 7M |
| Unit-III |  |  |  |  |  |
| 6 | a) | Draw the schematic diagram of the counter flow jet condenser and explain its working. | CO3 | L2 | 7M |
|  | b) | Explain the evaporative condenser with neat sketch. | CO3 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 7 | a) | Give a comparison between an impulse turbine and a reaction turbine. | CO3 | L2 | 7M |
|  | b) | In a Parson reaction turbine, the angles of receiving tips are 35º and of discharging tips 20º. The blade speed is 100 m/s. Calculate the tangential force, power developed, diagram efficiency and axial thrust of the turbine if its team consumption is 1 kg/min. | CO3 | L3 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Explain the Working principle of the simple vapour absorption refrigeration cycle. | CO4 | L1 | 7M |
|  | b) | A refrigerator works on the Bell-Coleman cycle and operates between pressure limits of 1.05 bar and 8.5 bar. Air is drawn from the cold chamber at 10ºC. Air coming out of the compressor is cooled to 30ºC before entering the expansion cylinder. Expansion and compression follow the law pv1.35=constant. Determine the theoretical COP of the system. | CO4 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | Calculate: i) Dew point temperature ii) Relative humidity iii) Specific humidity iv) Degree of saturation v) Enthalpy of mixture per kg of air when the dry bulb temperature is 30º C, wet bulb temperature is 20º C and the barometer reads 740 mm of Hg. | CO4 | L3 | 7M |
|  | b) | Explain the difference between winter air conditioning and summer air conditioning. | CO4 | L2 | 7M |