20ME403

Hall Ticket Number:

| II/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July/August, 2023 | Mechanical Engineering |  |  |
| Fourth Semester | Kinematics of Machines |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | What is the degree of freedom of a mechanism? How is it determined? | CO1 | L1 | 1 |
|  | b) | What is the difference between mechanism and machine? | CO1 | L1 | 1 |
|  | c) | What is the main advantage of hart mechanism over the Peaucellier mechanism? | CO1 | L1 | 1 |
|  | d) | Define ‘Coriolis component of acceleration’ | CO2 | L1 | 1 |
|  | e) | Explain the concept of relative velocity in mechanisms. | CO2 | L2 | 1 |
|  | f) | What is the relation between number of pairs (p), forming kinematic chain and number of pairs(n) | CO2 | L1 | 1 |
|  | g) | Deduce the expressions for finding maximum velocity and acceleration for simple harmonic motion follower. | CO3 | L1 | 1 |
|  | h) | Distinguish radial and off set followers used in cam mechanism. | CO3 | L2 | 1 |
|  | i) | Differentiate cycloidal and Involute profile gear teeth. | CO4 | L2 | 1 |
|  | j) | Explain the reverted gear train. | CO4 | L2 | 1 |
|  | k) | Write all types of gear trains ? | CO4 | L1 | 1 |
|  | l) | Define instantaneous center and centrode | CO3 | L1 | 1 |
|  | m) | Differentiate Machine and structure? | CO1 | L2 | 1 |
|  | n) | State kennedy’s theorem. | CO2 | L1 | 1 |
| Unit-I |  |  |  |  |  |
| 2 | a) | Explain Grubler s ctiterian for determining degrees of freedom, how it is different from Kutzback criterian | CO1 | L2 | 7M |
|  | b) | Explain double slider crank chain and its inversions with neat sketches | CO1 | L2 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | Sketch and explain inversions of single slider crank chain | CO1 | L2 | 7M |
|  | b) | Discuss various types of constrained motion. | CO1 | L3 | 7M |
| Unit-II |  |  |  |  |  |
| 4 |  | A crank and rocker mechanism PQRS has the following dimensions: PQ= 375mm, QR=620mm, RS=500mm, PS=750mm, QL=310mm (L is an intermediate point) RM= 150mm, SM= 400mm (M is an offset point to link RS, take RMS in clock wise direction). PS is the fixed link. PQ is the crank rotates at 20 rad/sec clockwise and retard at 240 rad/sec2, when it makes an angle of 500 with the fixed link. Determine the linear accelerations of points, R, L and M and The angular velocity and accelerations of links QR and RS. | CO2 | L3 | 14M |
| (OR) |  |  |  |  |  |
| 5 |  | The dimensions of the four bar mechanism are AB = 50mm, BC = 66mm, CD = 56mm and AD= 100mm (fixed link). The link AB has an instantaneous angular velocity of 10.5 rad/s and aretardation of 26 rad/s2 in the counter clockwise direction. Find the angular accelerations of the links BC and CD. The crank angle is 60o. | CO2 | L3 | 14M |
| P.T.O 20ME403 Unit-III |  |  |  |  |  |
| 6 |  | A cam is required to give motion to a follower fitted with a roller 30mm in diameter. The lift of the follower is 35mm and is performed - With uniform acceleration for cam rotation 45 - With uniform retardation for cam rotation 60 The follower falls through immediately with simple harmonic motion while the cam turns 120. Then a period of dwell is followed for remaining rotation of the cam. The least radius of the cam is 25mm. The line of motion of the follower passes through the center of the cam axis. Construct the cam profile. | CO3 | L3 | 14M |
| (OR) |  |  |  |  |  |
| 7 |  | A cam with 30mm as minimum diameter is rotating at a uniform speed of 1200 rpm and has to give the following motion to a knife edged follower. a. Follower to complete out word stroke of 25mm during 120oof cam rotation with equal uniform acceleration and retardation. b. Follower to dwell for 60o of com rotation. c. Follower to return to its final position during 90o of cam rotation with SHM. d. Follower to dwell for the remaining 90o of cam rotation. Draw the cam profile if the axis of the roller follower passes through the axis of the cam. | CO3 | L3 | 14M |
| Unit-IV |  |  |  |  |  |
| 8 |  | In an epicyclic gear train, an arm carries two gears A and B having 36 and 45 teeth respectively. If the arm rotates at 150 r.p.m. in the anticlockwise direction about the centre of the gear A which is fixed, determine the speed of gear B. If the gear A instead of being fixed, makes 300 r.p.m. in the clockwise direction, what will be the speed of gear B? | CO4 | L3 | 14M |
| (OR) |  |  |  |  |  |
| 9 | a) | State and prove law of gearing. | CO4 | L2 | 7M |
|  | b) | Derive the expression for minimum No.of teeth on pinion in order to eliminate interference. | CO4 | L3 | 7M |