20EE401

Hall Ticket Number:

| II/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| August, 2022 | Electrical & Electronics Engineering |  |  |
| Fourth Semester | Analog Electronics |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | What is PN Junction. | CO1 | BL1 | 1M |
|  | b) | Define Ripple factor | CO1 | BL1 | 1M |
|  | c) | What is the purpose of filters | CO1 | BL1 | 1M |
|  | d) | Define clipping operation | CO1 | BL1 | 1M |
|  | e) | Define Transistor | CO2 | BL1 | 1M |
|  | f) | What is reverse leakage current | CO2 | BL1 | 1M |
|  | g) | Classify FETs | CO2 | BL1 | 1M |
|  | h) | Draw small signal model of FET | CO2 | BL1 | 1M |
|  | i) | Define feedback factor. | CO3 | BL1 | 1M |
|  | j) | What is Bark hausen criterion | CO3 | BL1 | 1M |
|  | k) | Draw the circuit of differential amplifier | CO4 | BL1 | 1M |
|  | l) | Mention the Linear applications of Op-amp | CO5 | BL1 | 1M |
|  | m) | Mention the Non Linear applications of Op-amp | CO5 | BL1 | 1M |
|  | n) | Draw the circuit of Zero-crossing detector | CO5 | BL1 | 1M |
| Unit-I |  |  |  |  |  |
| 2 | a) | Discuss about Current Components in a p-n junction diode with neat sketch | CO1 | BL2 | 7M |
|  | b) | A full wave rectifier produces an RMS voltage of 10V at 50Hz and feeds a resistance of 1.1KΩ and filter uses C=50µF. Find the ripple output voltage. | CO1 | BL3 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | Explain the operation of positive clamper | CO1 | BL2 | 7M |
|  | b) | Derive the expression for the ripple factor of inductor filter when used with Full- wave rectifier. | CO1 | BL2 | 7M |
| Unit-II |  |  |  |  |  |
| 4 | a) | Explain the operation of NPN transistor | CO2 | BL2 | 7M |
|  | b) | For a silicon Transistor, α=0.995, emitter current is 10mA & leakage current IC0=0.5μA. Find IC, IB, β, ICE0 . | CO2 | BL3 | 7M |
| (OR) |  |  |  |  |  |
| 5 | a) | Justify how MOSFET acts as an amplifier | CO2 | BL2 | 7M |
|  | b) | Draw V-I characteristics of n-channel JFET and Explain. | CO2 | BL2 | 7M |
| Unit-III |  |  |  |  |  |
| 6 | a) | Draw the basic block diagram of feedback amplifier and explain about it | CO3 | BL2 | 7M |
|  | b) | Derive the expression for frequency of RC phase shift oscillator | CO3 | BL3 | 7M |
| (OR) |  |  |  |  |  |
| 7 | a) | Compare ideal & practical characteristics of op-amp | CO4 | BL2 | 7M |
|  | b) | Design Wien Bridge oscillator for frequency of 1KHz | CO3 | BL2 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Draw the circuit of Inverting & Non-Inverting OP-amp with fed back and derive the expression for gain. | CO5 | BL2 | 7M |
|  | b) | Design an op-amp differentiator for an input frequency of 2KHz | CO5 | BL3 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | Draw the circuit of Schmitt Trigger and Explain about it | CO5 | BL3 | 7M |
|  | b) | Discuss about Precision rectifier with neat sketches | CO5 | BL2 | 7M |