20EE402

Hall Ticket Number:

| III/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| August,2022 | Electrical & Electronics Engineering |  |  |
| Fourth Semester | Digital Electronics |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | What is an acceptable low level voltage for a TTL IC?. | CO1 | L1 | 1M |
|  | b) | Find the binary equivalent of (374.26)8. | CO1 | L1 | 1M |
|  | c) | Obtain the truth table for EX-NOR gate. | CO1 | L1 | 1M |
|  | d) | What is the difference between Latch and Flip-flop? | CO2 | L1 | 1M |
|  | e) | What are the applications of Octal number system? | CO1 | L1 | 1M |
|  | f) | What are the Limitations of Karnaugh maps? | CO2 | L1 | 1M |
|  | g) | Define Pair, Quad, and Octet? | CO2 | L1 | 1M |
|  | h) | Define Fan-in and Fan-out | CO3 | L1 | 1M |
|  | i) | What is Half-adder? | CO2 | L1 | 1M |
|  | j) | What is De-Multiplexer? | CO2 | L1 | 1M |
|  | k) | Write any differences between Combinational Circuits & Sequential Circuits? | CO4 | L1 | 1M |
|  | l) | Define Rise-time? | CO4 | L1 | 1M |
|  | m) | List the different types PLDs. | CO5 | L1 | 1M |
|  | n) | What is the difference between PALs and PLAs? | CO6 | L1 | 1M |
| Unit-I |  |  |  |  |  |
| 2 | a) | Perform the subtraction (11010- 1101) with the following binary numbers using (i) 2’s complement and (ii) l’s complement. Check the answer by straight subtraction. | CO1 | L2 | 7M |
|  | b) | Construct the hamming code for the data bits 1101 | CO1 | L2 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | Convert the following into canonical SOP (i) AB + B′C + A′B′C′ (ii) BC + AC′ + AB | CO1 | L3 | 7M |
|  | b) | Briefly explain about TTL logic gate? | CO1 | L2 | 7M |
| Unit-II |  |  |  |  |  |
| 4 | a) | Simplify f(w, x, y, z) = ∑(0, 1, 2, 8, 12, 13, 14) + d(3, 5, 10, 15) with K-Map. | CO2 | L3 | 7M |
|  | b) | Write short notes on Parity generator and checker | CO2 | L2 | 7M |
| (OR) |  |  |  |  |  |
| 5 | a) | Simplify the following Boolean function by using the tabulation method: F = ∑(0, 1, 2, 8, 10, 11, 14, 15) | CO2 | L2 | 7M |
|  | b) | Implement a combinational circuit which convert four bit excess 3 code to four bit BCD code. | CO2 | L2 | 7M |
| Unit-III |  |  |  |  |  |
| 6 | a) | Covert T flip-flip to D flip-flop. | CO3 | L2 | 7M |
|  | b) | Draw the diagram of a 3-bit binary Asynchronous down-counter . | CO3 | L2 | 7M |
| (OR) |  |  |  |  |  |
| 7 | a) | Briefly explain master slave JK Flip-flop. | CO4 | L2 | 7M |
|  | b) | Design a MOD 10 synchronous binary up counter using T flip-flop and other necessary logic gates. | CO4 | L3 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Explain R-2R ladder D/A converter. | CO5 | L2 | 7M |
|  | b) | Describe the working of a successive approximation A/D converter with the help of a suitable diagram | CO5 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | How does the PLA differ from ROM from the point of view of functionality | CO6 | L2 | 7M |
|  | b) | Describe the differences between the PLA and PAL. | CO6 | L2 | 7M |