20EE405

Hall Ticket Number:

| II/IV B.Tech(Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| August,2022 | Electrical and Electronics Engineering |  |  |
| Fourth Semester | Generation & Transmission |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | Define plant use factor?. | CO1 | L1 | 1M |
|  | b) | List the different types of tariffs. | CO1 | L1 | 1M |
|  | c) | Give the reasons for low power factor. | CO1 | L1 | 1M |
|  | d) | What is load duration curve? | CO2 | L1 | 1M |
|  | e) | Write the function of condenser in thermal power plant. | CO2 | L1 | 1M |
|  | f) | What is the role of penstock in hydro power plant? | CO2 | L1 | 1M |
|  | g) | What is Nuclear fission? | CO3 | L1 | 1M |
|  | h) | Define solar cell? | CO3 | L1 | 1M |
|  | i) | Define GMD&GMR? | CO3 | L1 | 1M |
|  | j) | Classify the overhead transmission lines? | CO4 | L1 | 1M |
|  | k) | What is the effect of ground on capacitance? | CO4 | L1 | 1M |
|  | l) | Define string efficiency? | CO4 | L1 | 1M |
|  | m) | Define sag? | CO2 | L1 | 1M |
|  | n) | What is arcing ground? | CO4 | L1 | 1M |
| Unit-I |  |  |  |  |  |
| 2 | a) | Briefly explain the methods of improving the Power Factor? | CO1 | L2 | 7M |
|  | b) | A generating station has the following daily load cycle : Time (Hours); 0—6; 6—10; 10—12; 12—16; 16—20; 20—24. Load (MW); 40; 50; 60; 50; 70; 40. Draw the load curve and Determine (i) maximum demand (ii) units generated per day (iii) average load and (iv) load factor. | CO1 | L2 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | Explain two part tariff and three part tariff methods. | CO1 | L2 | 7M |
|  | b) | An electric supply company having a maximum load of 50 MW generates 18 × 10 7 units per annum and the supply consumers have an aggregate demand of 75 MW. The annual expenses including capital charges are : For fuel = Rs 90 lakhs Fixed charges concerning generation = Rs 28 lakhs Fixed charges concerning transmission= Rs 32 lakhs and distribution Assuming 90% of the fuel cost is essential to running charges and the loss in transmission and distribution as 15% of kWh generated, deduce a two part tariff to find the actual cost of supply to the consumers. | CO1 | L3 | 7M |
| Unit-II |  |  |  |  |  |
| 4 | a) | What are the factors considered for the selection of site for hydro-electric power plants? | CO2 | L3 | 7M |
|  | b) | Draw and explain the principle of operation of nuclear reactor with a neat sketch. | CO2 | L2 | 7M |
| (OR) |  |  |  |  |  |
| 5 |  | Explain the layout of thermal power plant with function of each block in it. | CO2 | L2 | 14M |
| Unit-III |  |  |  |  |  |
| 6 | a) | Derive the expression for voltage regulation in case of short transmission line with the help of phasor diagram. | CO3 | L2 | 7M |
|  | b) | A single phase overhead transmission line delivers 1100 kW at 33 kV at 0•8 p.f. lagging. The total resistance and inductive reactance of the line are 10 Ω and 15 Ω respectively. Determine: (i) sending end voltage (ii) sending end power factor and (iii) transmission efficiency | CO3 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 7 | a) | Derive an expression for Inductance of a Single Phase Two-wire Line. | CO3 | L2 | 7M |
|  | b) | A 3-phase, 50 Hz, 132 kV overhead line has conductors placed in a horizontal plane 4 m apart. Conductor diameter is 2 cm. If the line length is 100 km, calculate the charging current per phase assuming complete transposition. | CO3 | L3 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Explain about pin type and suspension type of Insulator with neat diagrams. | CO4 | L2 | 7M |
|  | b) | Derive the expression for sag when the conductor is placed between two unequal supports | CO4 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | Derive the expression for voltage wave and current wave in the concept of travelling waves. | CO4 | L2 | 7M |
|  | b) | Derive the expression for critical disruptive voltage when corona occurs on a transmission line. | CO4 | L3 | 7M |