18ME502

Hall Ticket Number:

| III/IV B.Tech (Regular / Supplementary) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| January, 2022 | Mechanical Engineering |  |  |
| Fifth Semester | I.C.Engines & Gas Turbines |  |  |
| Time: Three Hours |  | Maximum : 50 Marks |  |
| Answer Question No.1 compulsorily. |  |  | (1X10 = 10 Marks) |
| Answer ONE question from each unit. |  |  | (4X10=40 Marks) |

1. Answer all questions                                                                                                                          (1X10=10 Marks)

| a) Define clearance volume. | CO1 |
| --- | --- |
| b) Define stoichiometric air fuel ratio. | CO1 |
| c) Sketch Otto cycle. | CO1 |
| d) Name different methods to measure frictional power. | CO2 |
| e) What is the significance of specific fuel consumption? | CO2 |
| f) What is the major constituent of natural gas? | CO3 |
| g) List various alcohol fules? | CO3 |
| h) List out main parts of gas turbine. | CO4 |
| i) Define thrust. | CO4 |
| j) Write few applications of gas turbine. | CO4 |

UNIT – I

| 2. a) | Explain the working of four stroke spark ignition engine with the help of neat sketches. | CO1 | 5 M |
| --- | --- | --- | --- |
| b) | Compare the advantages and disadvantages of four stroke and two stroke engines | CO1 | 5 M |

(OR)

| 3. a) | With a neat sketch explain the working principle of a simple carburetor. | CO1 | 5 M |
| --- | --- | --- | --- |
| b) | Describe briefly the MPFI system with a neat sketch. | CO1 | 5 M |

UNIT – II

| 4. a) | Briefly explain the stages of combustion in compression ignition engines. | CO2 | 5 M |
| --- | --- | --- | --- |
| b) | Explain the phenomenon of knock in SI engines and compare it with CI engine knock. | CO2 | 5 M |

(OR)

| 5.a) | Explain Indicated power, Brake power and Specific fuel consumption. | CO2 | 3 M |
| --- | --- | --- | --- |
| b) | During the trial on single cylinder, four stroke oil engine, the following results were obtained. Cylinder diameter 20 cm, stroke 40 cm, mean effective pressure 6 bar, torque 407 Nm, speed 250 rpm, oil consumption 4 kg/h, Calorific value of fuel 43 MJ/kg, Cooling water flow rate 4.5 kg/min, air used per kg of fuel 30 kg, rise in cooling water temperature 45oC, temperature of exhaust gas 420oC, room temperature 20oC, mean specific heat of exhaust gas 1 kJ/kg.K, and specific heat of water 4.18 kJ/kg.K. Find the indicated power , brake power and draw up a heat balance sheet for ht test in kJ/h | CO2 | 7 M |

UNIT – III

| 6.a) | Briefly explain the necessity for alternate fuels. | CO3 | 5 M |
| --- | --- | --- | --- |
| b) | Write a brief note on the following alternate fuels. (i) LPG & (ii) CNG | CO3 | 5 M |

(OR)

| 7.a) | What are the various emissions from CI engine- Explain | CO3 | 5 M |
| --- | --- | --- | --- |
| b) | Explain the working of Catalytic converter. | CO3 | 5 M |

P.T.O.

18ME502

UNIT – IV

| 8.a) | Explain the working of open cycle Gas turbine with the help of line diagram and T – s diagram. | CO4 | 5 M |
| --- | --- | --- | --- |
| b) | The air supplied to a gas turbine plant is 10kg/s. The pressure ratio is 6 and pressure at the inlet of compressor is 1 bar. The compressor is two stage and is provided with perfect inter-cooling. The inlet temperature is 300 K and maximum temperature is limited to 1073 K. Take isentropic efficiencies of compressor and turbine a 80% and 85% respectively. Neglect the mass of fuel and calculate the thermal efficiency of the plant. | CO4 | 5 M |

(OR)

| 9.a) | Explain the working of turbojet engine with neat sketch. | CO4 | 5M |
| --- | --- | --- | --- |
| b) | State the fundamental difference between the rocket propulsion and jet propulsion | CO4 | 5M |