18ME501

Hall Ticket Number:

| II/IV B.Tech (Regular)DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| February, 2021 | Mechanical Engineering |  |  |
| Fourth Semester | Machine Dynamics |  |  |
| Time: Three Hours |  | Maximum: 60 Marks |  |
| Answer ALL Questions from PART-A. |  |  | (1X12 = 10 Marks) |
| Answer ANY FOUR questions from PART-B. |  |  | (4X12=40 Marks) |
|  |  |  | PART-A |

| 1. | a) | State D’Alembert’s principle. |  |
| --- | --- | --- | --- |
|  | b) | What do you mean by piston effort? |  |
|  | c) | Differentiate between a flywheel and governor. |  |
|  | d) | When do you say that a governor is isochronous? |  |
|  | e) | What do you mean by angle of heel? |  |
|  | f) | Differentiate between static and dynamic balancing. |  |
|  | g) | What do you mean by resonance? |  |
|  | h) | Define damping ratio. |  |
|  | i) | What is a seismic unit? |  |
|  | j) | Define whirling or whipping speed of a shaft. |  |
| PART-B |  |  |  |
| 2. | a) | Derive expressions for velocity and acceleration of a piston in slider crank mechanism. | 4M |
|  | b) | A vertical petrol engine 100 mm diameter and 125 mm stroke has a connecting rod 250 mm long. The weight of the piston is 12 N. The speed is 2000 rpm. On the expansion stroke, with the crank 20o from the top dead centre, the gas pressure is 700 kN/m2. Determine: - net force on the piston - resultant load on the gudgeon pin - thrust on the cylinder walls - crank pin effort | 6M |
| 3. | a) | What is a controlling force curve? How is it useful in analyzing the performance of a governor? | 4M |
|  | b) | The upper arms of a porter governor are pivoted on the axis of rotation and the lower arms are pivoted to the sleeve at a distance of 30 mm from the axis of rotation. The length of each arm is 300 mm, and the mass of each ball is 6 kg. If the equilibrium speed is 200 rpm, when the radius of rotation is 200 mm, find the required mass on the sleeve. If the friction is equivalent to a force of 40 N at the sleeve, find the coefficient of insensitiveness at 200 mm radius. | 6M |
| 4. | a) | What is gyroscopic couple? Derive an expression for the same. | 4M |
|  | b) | The turbine rotor of a ship of mass 6000 kg has a radius of gyration of 500 mm. It rotates at 1820 rpm, clockwise when viewed from the stern. Determine the gyroscopic effect on the ship when, the ship is pitching, and the bow is descending with maximum velocity. The pitching is simple harmonic, periodic time being 20 seconds, and the total angular movement between extreme positions is 10o. | 6M |
| 5. |  | A shaft has three eccentrics, each 75 mm diameter and 25 mm thick, machined in one piece with the shaft. The central planes of the eccentric are 60 mm apart. The distance of the centers from the axis of rotation are 12 mm, 18 mm and 12 mm and their angular positions are 120o apart. The density of the metal is 7000 kg/m3. If the shaft is balanced by adding two masses at a radius 75 mm and at distances of 100 mm from the central plane of the middle eccentric, find the amount of masses and their angular positions. | 10M |
| P.T.O. 18ME501 |  |  |  |
| 6. | a) | Derive the differential equation of motion of a spring-mass system and find out its natural frequency. | 4M |
|  | b) | Find the mass of the given spring mass system shown in Fig. Where k1=2000N/m, k2=1500 N/m, k3=3000 N/m and k4=k5=500N/m. The natural frequency of the system is of 10 Hz. | 6M |
| 7. | a) | What is logarithmic decrement? Derive an expression for the same. | 4M |
|  | b) | A spring –mass-damper system has a mass of 3 kg with stiffness 100 N/m and damping constant of 3 N-s/m. Determine i) Damping factor, ii) Natural frequency of damped vibration, iii) Logarithmic decrement, iv) Ratio of two consecutive amplitudes | 6M |
| 8. | a) | A vehicle has a mass of 490 kg and the total spring constant of its suspension system is 58800 N/m. The profile of the road may be approximated to a sine wave of amplitude 40 mm and wave length 4.0 m. Determine i) the critical speed of the vehicle, ii) the amplitude of steady state motion of the mass when the vehicle is driven at critical speed and the damping factor is 0.5, and iii) the amplitude of steady state motion of the mass when the vehicle is driven at 57 km/hr and the damping factor is 0.5. | 5M |
|  | b) | A machine of 100 kg mass is supported on springs of total stiffness 700 kN/m and has an unbalanced rotating element, which results in a disturbing force of 350 N at a speed of 3000 rpm. Assuming a damping factor of 0.20, determine i) its amplitude of motion due to unbalance, ii) the transmissibility, and iii) the transmitted force. | 5M |
| 9. | a) | Derive an expression for transmissibility ratio. | 5M |
|  | b) | Explain the working principle of vibration measuring instruments | 5M |