18EE504

Hall Ticket Number:

| III/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| February, 2021 | Electrical and Electronics Engineering |  |  |
| Fifth Semester | Microprocessors and Microcontrollers |  |  |
| Time: Three Hours |  | Maximum: 50 Marks |  |
| Answer question 1 compulsory. |  |  | (10X1 = 10Marks) |
| Answer one question from each unit. |  |  | (4X10=40Marks) |

| 1. |  | Answer all questions | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
|  | a) | What is the maximum capacity of the memory that can be interfaced to the microprocessor which contains 20 address lines? | CO1 | L1 | 1M |
|  | b) | What will be 20-bit physical address form which the code is accessed? If [CS]=348AH, [IP]=4214H. | CO1 | L2 | 1M |
|  | c) | What will be the contents of DL after execution of the instruction ROL DL,01H? If [DL]=EFH and carry flag is reset. | CO1 | L2 | 1M |
|  | d) | Which of the modes of 8254 timer generates square wave? | CO2 | L1 | 1M |
|  | e) | In order to program Port A and Port B as output ports in Mode 0 and Port C as input port in Mode 0, the control word for 8255 would be | CO2 | L2 | 1M |
|  | f) | The 8051 has ________ 16-bit counter/timers. | CO3 | L1 | 1M |
|  | g) | Find the number of times the following loop will be executed MOV R6,#100 BACK:MOV R5,#100 HERE:DJNZ R5,HERE DJNZ R6,BACK END | CO4 | L3 | 1M |
|  | h) | What is the total amount of external code memory that can be interfaced to the 8051 | CO3 | L1 | 1M |
|  | i) | Name two 16-bit registers of 8051 microcontroller | CO3 | L1 | 1M |
|  | j) | What is the functionality of ULN2003 IC in interfacing stepper motor with 8051 microcontrollers | CO4 | L1 | 1M |
| UNIT-I |  |  |  |  |  |
| 2. | a) | Draw the architecture of 8086 Microprocessor and Explain the functionality of each block. | CO1 | L2 | 5M |
|  | b) | Explain about different types of Addressing modes in 8086 with examples | CO1 | L2 | 5M |
| (OR) |  |  |  |  |  |
| 3. | a) | Explain about following instructions. (a) OUT (b) MUL (c) CMP (d) MOVSB | CO1 | L2 | 5M |
|  | b) | Explain about program development tools used in assembly language programming | CO1 | L2 | 5M |
| UNIT-II |  |  |  |  |  |
| 4. | a) | Explain about 8255 Programmable peripheral interfacing device with a neat sketch | CO2 | L2 | 5M |
|  | b) | Explain different modes in which 8255 is operated in connecting peripheral devices to 8086 | CO2 | L2 | 5M |
| (OR) |  |  |  |  |  |
| 5. | a) | Explain how a matrix keyboard is interfaced with 8086 microprocessors with the help of a neat sketch. | CO2 | L2 | 5M |
|  | b) | Write in detail about modes of operation of 8254 | CO2 | L2 | 5M |
| UNIT-III |  |  |  |  |  |
| 6. | a) | Write about the comparison of microprocessors with microcontrollers | CO3 | L2 | 5M |
|  | b) | Draw the internal architecture of 8051 and explain different blocks. | CO3 | L3 | 5M |
| (OR) |  |  |  |  |  |
| 7. | a) | Explain about memory organization of 8051 microcontrollers | CO3 | L2 | 5M |
|  | b) | Explain about I/O port programming in 8051 microcontrollers | CO3 | L3 | 5M |
| UNIT-IV |  |  |  |  |  |
| 8. | a) | Explain about interfacing of ADC with 8051 microcontrollers using a diagram | CO4 | L3 | 5M |
|  | b) | Explain about timer port programming in 8051 microcontrollers | CO4 | L2 | 5M |
| (OR) |  |  |  |  |  |
| 9. | a) | Explain how a stepper motor is interfaced to an 8051 µc with a neat diagram. | CO4 | L2 | 5M |
|  | b) | Draw the format of (i) TMOD (ii) SCON special function registers in 8051 microcontrollers. | CO4 | L3 | 5M |