20ME602

Hall Ticket Number:

| III/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July/August, 2023 | Mechanical Engineering |  |  |
| Sixth Semester | Design of Transmission Elements |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | What type of stresses is induced in shafts? | CO1 | L2 | 1M |
|  | b) | How the shaft is designed when it is subjected to twisting moment only? | CO1 | L2 | 1M |
|  | c) | How are the keys classified? State their applications. | CO1 | L1 | 1M |
|  | d) | How does the working of a clamp coupling differ from that of a muff coupling? Explain | CO1 | L2 | 1M |
|  | e) | What are rolling contact bearings? | CO2 | L1 | 1M |
|  | f) | Write the various types of lubrications. | CO2 | L1 | 1M |
|  | g) | Write the various properties of lubricants. | CO2 | L1 | 1M |
|  | h) | How does the function of a brake differ from that of a clutch | CO3 | L2 | 1M |
|  | i) | What are the advantages of V-belt drive over flat belt drive? | CO3 | L2 | 1M |
|  | j) | What is meant by a clutch? | CO3 | L1 | 1M |
|  | k) | What is the main function of a flywheel in an engine? | CO4 | L1 | 1M |
|  | l) | Discuss the various types of stresses induced in a flywheel rim. | CO4 | L2 | 1M |
|  | m) | Define a gear? | CO4 | L1 | 1M |
|  | n) | Explain how the gears are classified? Name the various types of gears. | CO4 | L2 | 1M |
| Unit-I |  |  |  |  |  |
| 2 | a) | Write the applications of split muff couplings? | CO1 | L1 | 4M |
|  | b) | Design a muff coupling to connect two shafts transmitting 40KW at 120rpm. The permissible shear and crushing stress for the shaft and key material (mild steel) are 30Mpa and 80Mpa respectively. The material of muff is cast Iron with permissible shear stress of 15Mpa. Assume that the maximum torque transmitted is 25 percent greater than mean torque. | CO1 | L4 | 10M |
|  |  | (OR) |  |  |  |
| 3 | a) | A 10kW power is transmitted at 800 rpm, from a motor shaft, through a key, to a machine shaft by a means of a pulley and a belt. Design the key. Take the allowable shear stress and crushing stress are 45MPa and 100Mpa. | CO1 | L4 | 6M |
|  | b) | A shaft is required to transfer 43kW of power at 600rpm. The outside diameter must not exceed 50mm and the maximum shear stress is not to exceed 70N/mm2. Find out the dimensions of hollow and solid shaft, which would meet these requirements. | CO1 | L3 | 8M |
| Unit-II |  |  |  |  |  |
| 4 |  | A shaft, 150mm in diameter, rotates in a bearing at 2000rpm. The length of the bearing is 1.4 times its diameter. The bearing pressure is 1N/mm2, and the coefficient of friction at the bearing surface is 0.005. Determine the power loss in friction. The temperature of the bearing is entirely controlled by the flow of oil through the bearing. If the difference between the outlet and inlet temperatures is 15 0C, determine the quantity of coolant oil required, if the specific heat of the oil is 1900 J/kg/ o C. | CO2 | L3 | 14M |
| (OR) |  |  |  |  |  |
| 5 | a) | State any four advantages of Rolling contact bearings over sliding contact bearings. | CO2 | L2 | 4M |
|  | b) | A rolling contact bearing is subjected to the following work cycle: (a) Radial load of 6000N at 150 r.p.m for 25% of the time; (b) Radial load of 7500N at 600 r.p.m for 20% of the time; and (c) Radial load of 2000N at 300 r.p.m for 55% of the time. The inner ring rotates and loads are steady .Select a bearing for an expected average life of 2500 hours. | CO2 | L4 | 10M |
| P.T.O 20ME602 Unit-III |  |  |  |  |  |
| 6 |  | A V- belt is to transmit 20 kW from a 250 mm pitch diameter sheave to a 900mm diameter pulley. The centre distance between the two shafts is 1000 mm. The groove angle is 40 degree and the coefficient of friction between the belt and the sheave and also between the belt and the pulley is 0.2. The cross-section of the belt is 40 mm wide at the top, 20 mm wide at the bottom and 25 mm deep. The density of the belt is 1000 kg/m3 and the allowable tension per belt is 1000 N. Find the number of belts required. | CO3 | L3 | 14M |
| (OR) |  |  |  |  |  |
| 7 | a) | A multiple disc clutch, steel on bronze, is to transmit 4.5 kW at 750 r.p.m. The inner radius of the contact is 40 mm and outer radius of the contact is 70 mm. The clutch operates in oil with an expected coefficient of 0.1. The average allowable pressure is 0.35 N/mm2. Find : 1. The total number of steel and bronze discs; 2. the actual axial force required and 3. the actual maximum pressure. | CO3 | L3 | 10M |
|  | b) | Discuss the different types of brakes giving atleast one practical application for each. | CO3 | L2 | 4M |
| Unit-IV |  |  |  |  |  |
| 8 |  | The intercepted areas between the output torque curve and the mean resistance line of a turning moment diagram for a multicylinder engine, taken in order from one end are as follows: – 35, + 410, – 285, + 325, – 335, + 260, – 365, + 285, – 260 mm2. The diagram has been drawn to a scale of 1 mm = 70 N-m and 1 mm = 4.5°. The engine speed is 900 r.p.m. and the fluctuation in speed is not to exceed 2% of the mean speed. Find the mass and cross-section of the flywheel rim having 650 mm mean diameter. The density of the material of the flywheel may be taken as 7200 kg / m3. The rim is rectangular with the width 2 times the thickness. Neglect effect of arms, etc. | CO4 | L3 | 14M |
| (OR) |  |  |  |  |  |
| 9 |  | A pair of straight teeth spur gears, having 20° involute full depth teeth is to transmit 12 kW at 300 r.p.m. of the pinion. The speed ratio is 3: 1. The allowable static stresses for gear of cast iron and pinion of steel are 60 MPa and 105 MPa respectively. Assume the following: Number of teeth of pinion = 16; Face width = 14 times module; Determine the module, face width and pitch diameter of gears. Check the gears for wear; given σes = 600 MPa; EP = 200 kN/mm2 and EG = 100 kN/mm2. | CO4 | L4 | 14M |