20EE603

Hall Ticket Number:

| III/IV B.Tech (Regular) DEGREE EXAMINATION |  |  |  |
| --- | --- | --- | --- |
| July/August, 2023 | Electrical and Electronics Engineering |  |  |
| Sixth Semester | Electrical Drives |  |  |
| Time: Three Hours |  | Maximum: 70 Marks |  |
| Answer question 1 compulsory. |  |  | (14X1 = 14Marks) |
| Answer one question from each unit. |  |  | (4X14=56 Marks) |

|  |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| 1 | a) | What are the advantages of electric drives | CO1 | L1 | 1 |
|  | b) | Classify electric drives | CO1 | L2 | 1 |
|  | c) | What is the importance of control unit in an electric drive system? | CO1 | L1 | 1 |
|  | d) | Relate fundamental torque of a variable inertia drive system with various parameters | CO2 | L1 | 1 |
|  | e) | Why regenerative braking is not possible in series motor? | CO2 | L1 | 1 |
|  | f) | Compare circulating current and non-circulating current mode of operation of a dual converter drive | CO2 | L2 | 1 |
|  | g) | List the drawbacks of rectifier fed dc drives | CO2 | L1 | 1 |
|  | h) | Name the control strategies of chopper | CO1 | L1 | 1 |
|  | i) | Identify different methods of speed control of induction motor | CO3 | L1 | 1 |
|  | j) | What are the applications of stator voltage control scheme? | CO3 | L1 | 1 |
|  | k) | What is the need of the external rotor resistances during plugging of an induction motor? | CO3 | L1 | 1 |
|  | l) | Define self-control mode of synchronous motor | CO4 | L1 | 1 |
|  | m) | What are major applications of brushless dc motor? | CO4 | L1 | 1 |
|  | n) | Name the various braking methods used in synchronous motors | CO4 | L1 | 1 |
| Unit-I |  |  |  |  |  |
| 2 | a) | Outline the block diagram of basic electrical drive system. Explain each component | CO1 | L2 | 7M |
|  | b) | A drive has the following parameters: T=150-0.1N, N-m, where N is the speed in rpm, Load Torque Tl=100, N-m. Initially the drive is operating in steady state. The Characteristics of the load torque are changed to Tl= -100, N-m. Calculate initial and final equilibrium speeds. | CO1 | L3 | 7M |
|  |  | (OR) |  |  |  |
| 3 | a) | Derive the fundamental torque expression of translational motor load system | CO1 | L3 | 7M |
|  | b) | A mass of 500 kg is being lifted at uniform speed of 1.5 m/s by a winch driven by a motor running at a speed of 1000 r/min. The moments of inertia of the motor and winch are 0.5 and 0.3 kg-m2, respectively. In the absence of the mass, the motor develops a torque of 100 Nm at a speed of 1000 rpm. Calculate a) The equivalent moment of inertia referred to the motor side b) The motor torque. | CO1 | L3 | 7M |
| Unit-II |  |  |  |  |  |
| 4 | a) | Explain with neat waveforms the operation of a single phase half controlled fed dc separately excited motor | CO2 | L2 | 7M |
|  | b) | A 220 V, 24 A, 1000 rpm separately excited dc motor having an armature resistance of 2 Ω is controlled by a chopper. The chopping frequency is 500 Hz and the input voltage is 230 V. Calculate the duty ratio for a motor torque of 1.2 times rated torque at 500 rpm | CO2 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 5 | a) | Compare conventional and chopper fed dc drives | CO2 | L2 | 7M |
|  | b) | A 230 V, 960 rpm and 200 A separately excited DC motor has an armature resistance of 0.02Ω.The motor is fed from a chopper which provides both motoring and braking operations. The source has a voltage of 230 V DC. Assuming continuous conduction. - Calculate duty ratio of chopper for motoring operation at rated torque and 350 rpm. - Calculate duty ratio of chopper for braking operation at rated torque and 350 rpm | CO2 | L3 | 7M |
| Unit-III |  |  |  |  |  |
| 6 | a) | Explain the pole amplitude modulation method of control in induction motor | CO3 | L2 | 7M |
|  | b) | A 440 V, 3-phase, 50 Hz, 6-pole, 945 rpm, delta connected induction motor has following parameters referred to the stator Rs = 2.0 Ω, Rr’ = 2.0 Ω, Xs = 3 Ω, Xr’ = 4Ω. When driving a fan load at rated voltage it runs at rated speed. The motor speed is controlled by stator voltage control, Determine (i) Motor terminal voltage, current and torque at 800 rpm. (ii) Motor speed, current and torque for the terminal voltage of 280 V | CO3 | L3 | 7M |
| (OR) |  |  |  |  |  |
| 7 | a) | Outline the speed torque curves with variable frequency control for two different modes. i) Operation at constant flux. ii) Operation at constant ( v/f ) ratio | CO3 | L2 | 7M |
|  | b) | A Y-connected squirrel cage induction motor has following ratings and parameters: 400V, 50Hz, 4-pole, 1370 rpm, Rs =2 Ω, Rr 1 = 3 Ω, Xs= Xr 1 =3.5 Ω, Xm=55 It is controlled by a current source inverter at a constant flux. Calculate: Motor torque, speed and stator current when operating at 30Hz and rated slip speed | CO3 | L3 | 7M |
| Unit-IV |  |  |  |  |  |
| 8 | a) | Explain the working principle of single-stack variable reluctance motor with a neat sketch | CO4 | L2 | 7M |
|  | b) | Examine the importance of vector control of synchronous motor drives | CO4 | L4 | 7M |
| (OR) |  |  |  |  |  |
| 9 | a) | Illustrate the operation of variable frequency control of multiple synchronous motors using VSI | CO4 | L2 | 7M |
|  | b) | Relate the performance of switched reluctance motor drive over other ac motor drives | CO4 | L2 | 7M |

| III/IV B.Tech (Regular\Supplementary) DEGREE EXAMINATION |  |  |
| --- | --- | --- |
| July, 2023 | Electrical and Electronics Engineering |  |
| Sixth Semester | Electrical Drives |  |
| Time: Three Hours |  | Maximum: 70 Marks |

Scheme of Evaluation

SET2

- a) advantages of electric drives 1M - Classify electric drives 1M - Importance of control unit in an electric drive system? 1M - fundamental torque of a variable inertia drive system with various parameters 1M - Why regenerative braking is not possible in series motor? 1M - Compare circulating current and non-circulating current mode 1M - drawbacks of rectifier fed dc drives 1M - Control strategies of chopper 1M - different methods of speed control of induction motor 1M - applications of stator voltage control scheme 1M - need of the external rotor resistances during plugging of an induction motor 1M - self-control mode of synchronous motor 1M - major applications of brushless dc motor 1M - various braking methods used in synchronous motors 1M - a) Block diagram of basic electrical drive system 4M Each component explanation 3M b) Data representation with necessary equations 2M solving the problem 5M - a) Representation 2M derivation 5M b) The equivalent moment of inertia referred to the motor calculation 4M The motor torque 3M - a) Representation 3M Explanation of single phase half controlled fed dc separately excited motor 4M b) Data representation with necessary equations 3M duty ratio calculation 4M - a) Comparison of any 5 points 7M b) Data representation with necessary formula 1M duty ratio of chopper for motoring operation 3M duty ratio of chopper for motoring operation 3M - a) pole amplitude modulation method explanation 7M b) Representation 1M Motor terminal voltage calculation 2M current calculation 2M torque calculation 2M - a) Operation at constant flux 4M Operation at constant ( v/f ) ratio 3M b) data representation 2M Motor torque calculation 2M speed and stator current calculation 3M - a) single-stack variable reluctance motor representation 3M working of single-stack variable reluctance motor 4M b) vector control of synchronous motor drives explanation 7M - a) Representation 3M operation using VSI 4M b) Any 5 points of comparison between switched reluctance motor drive over other ac motor drives 7M