14ME703

Hall Ticket Number:

| IV/IV B.Tech (Regular/Supplementary) DEGREE EXAMINATION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| January, 2021 |  |  | Mechanical Engineering |  |  |  |  |
| Seventh Semester |  |  | ENGINEERING METROLOGY & MECHANICAL MEASUREMENTS |  |  |  |  |
| Time: Three Hours |  |  |  | Maximum : 60 Marks |  |  |  |
| Answer ALL Questions from PART-A. |  |  |  |  | (1X12 = 12 Marks) |  |  |
| Answer ANY FOUR questions from PART-B. |  |  |  |  | (4X12=48 Marks) |  |  |
| Part - A |  |  |  |  |  |  |  |
| 1 | Answer all questions |  |  |  |  | (1X12=12 Marks) |  |
|  | a) | What do you understand by line and end standards? |  |  |  |  |  |
|  | b) | Differentiate tolerance and allowance |  |  |  |  |  |
|  | c) | Explain Taylors’s principle for the design of limit gauges |  |  |  |  |  |
|  | d) | Explain the working of spirit levels. |  |  |  |  |  |
|  | e) | Differentiate primary and secondary texture |  |  |  |  |  |
|  | f) | Name the important elements of screw thread. |  |  |  |  |  |
|  | g) | What are the Static performance characteristics of a measurement system? |  |  |  |  |  |
|  | h) | What is the purpose of strain rosette? |  |  |  |  |  |
|  | i) | Write explanatory notes on Hot-wire anemometer. |  |  |  |  |  |
|  | j) | Classify pressure measurement techniques. |  |  |  |  |  |
|  | k) | Explain the principle of thermisters. |  |  |  |  |  |
|  | l) | Define Load cell and state its applications |  |  |  |  |  |
| Part - B |  |  |  |  |  |  |  |
| 2 | a) | Differentiate i) Unilateral and bilateral tolerances ii) clearance fit and interference fit iii) tolerance and allowance. |  |  |  |  | 6M |
|  | b) | Discuss in briefly shaft and hole basis system of fits. Which is preferred. Why? |  |  |  |  | 6M |
| 3 | a) | Calculate the dimensions of plug and ring gauges to control the production of 50mm shaft and hoe pair of H7d8 fit. The following assumptions may be made. 50 mm falls in the diameter step of 30- 50 mm. Upper deviation for ‘d’ shaft is -16D0.44. IT7 is 16i and IT8 is 25i, gauge tolerance=10% work tolerance and wear allowance=10% of gauge tolerance. |  |  |  |  | 12M |
| 4 | a) | Describe any one of the mechanical comparators. How will you distinguish between mechanical and optical comparators? |  |  |  |  | 6M |
|  | b) | What is flatness? What are the various methods of checking flatness of surfaces? |  |  |  |  | 6M |
| 5 | a) | What is meant by effective diameter of a screw thread? Explain with a neat sketch how the same can be measured using 3 wire method. |  |  |  |  | 6M |
|  | b) | Explain with a neat sketch how the same can be measured using 2 wire method. |  |  |  |  | 6M |
| 6 | a) | With the help of a block diagram, show the three stages of Generalized Measurement System. Also explain the functions of each stage. |  |  |  |  | 6M |
|  | b) | Derive, from first principles, the relationship for gauge factor of a strain gauge. |  |  |  |  | 6M |
| 7 | a) | Why Rotameter is called constant head variable area flow meter? Explain the working rota meter with neat sketch. |  |  |  |  | 6M |
|  | b) | With a neat sketch explain the working of hot wire anemometer. |  |  |  |  | 6M |
| 8 | a) | Explain with a neat sketch the constructional features and basic working principle of McLeod gauges used for measurement of low pressures. |  |  |  |  | 6M |
|  | b) | Classify temperature measuring instruments and explain working of thermocouple for the measurement of temperature |  |  |  |  | 6M |
| 9 | a) | Explain the construction and working of pneumatic load cell |  |  |  |  | 6M |
|  | b) | Explain the principle of measuring shaft torques using rope brake dynamometer. |  |  |  |  | 6M |